Telemetry & Real-Time Analytics :

## Overall Flow (Use Case)

| ## Telemetry & Real-Time Analytics |  |  |
| --- | --- | --- |
| ## Use Case: | ## Edge Data Filtering | ## Implementing logic to decide what data stays on the car and what goes to the cloud (e.g., sending GPS every second, but transmission temperature only if it exceeds a threshold). |

- AWS Cloud requires a paid subscription; therefore, Firebase is used as the cloud backend instead of AWS.

- First, we sign in to Firebase after creating a project and database in the Firebase console.

- The Firebase database is then linked to our PC.

- The Firebase backend setup is completed successfully, and PC-to-Firebase connectivity is validated.

- The temperature sensor measures the current temperature.

- The ESP32 reads this value continuously.

- ESP32 connects to Wi-Fi and sends the temperature to Firebase.

- Firebase stores the value in the Realtime Database.

- A threshold of 30°C is set in the code.

- If the temperature is below 30°C, no alert is generated.

- If the temperature exceeds 30°C, an alert flag becomes true.

- This alert is updated instantly in Firebase.

- The system works in real time.

- It shows how sensor data can trigger cloud alerts automatically.

- Next, we will connect the OKT507-C board to the PC and link it to our Firebase project.

- Finally, the OKT507-c board fetches the temperature values from the firebase realtime database.

## Firebase Creation:
Step-by-Step Implementation Using Firebase

### Create Firebase Project

- Go to Firebase Console

- Create a new project

- Enable Realtime Database

### Set database rules (for testing):

### STEP 1: Create Firebase Project

#### 1️⃣ Click the big box:

“Get started by setting up a Firebase project”

(Highlighted in your screenshot)

### 🔹 STEP 2: Project Creation Screen

After clicking, you will see Create a project page.

#### 2️⃣ Enter Project name

Example (use this or your own):

edge-data-filtering

#### 3️⃣ Click Continue

### STEP 3: Google Analytics Screen

#### 4️⃣ You will see “Enable Google Analytics”

- Turn OFF (recommended for simplicity)

- Click Create project

Wait 10–20 seconds

#### 5️⃣ Click Continue once project is ready

## Firebase Project Created

Now you will be inside your Firebase project dashboard.

## NEXT IMPORTANT STEP: Enable Realtime Database

### STEP 4: Enable Realtime Database

#### 6️⃣ In left sidebar:

Click Build

#### 7️⃣ Click Realtime Database

#### 8️⃣ Click Create Database

### STEP 5: Database Setup

#### 9️⃣ Choose location:

- Select asia-southeast1 (best for India)

Click Next

### STEP 6: Set Rules (Testing Mode)

#### 🔟 Select:

Start in test mode

Click Enable

### STEP 7: Set Rules Manually

After database opens:

- Click Rules tab

- Replace everything with:

{

"rules": {

".read": true,

".write": true

}

}

- Click Publish

## STEP 8: Copy Database URL (IMPORTANT)

At the top of Realtime Database page, copy:

## https://console.firebase.google.com/project/edge-data-filtering/database/edge-data-filtering-default-rtdb/rules

Save this URL — we will use it in OKT507-C code.

## WHAT YOU HAVE DONE SO FAR

✔ Firebase project created
 ✔ Realtime Database enabled
 ✔ Database rules configured
 ✔ Database URL ready

### STEP 9: Get Database Secret (for OKT507-C)

Then:

- Send GPS data

- Apply edge filtering logic

- Push filtered data to Firebase

Before proceeding:

Tell me when you see “Realtime Database” screen
 or
 Send the next screenshot

# STEP 9: GET DATABASE SECRET (FOR OKT507-C)

This Database Secret is what allows your edge device (OKT507-C) to write data into Firebase using HTTP.

## STEP 9.1: Open Project Settings

1️⃣ In Firebase Console, look at the top-left corner
 2️⃣ Click the ⚙️ (Gear icon)
 3️⃣ Click Project settings

## STEP 9.2: Go to Service Accounts

4️⃣ In Project settings page, click the Service accounts tab
 5️⃣ Scroll down until you see Database secrets

## STEP 9.3: Generate Database Secret

6️⃣ Under Database secrets, you will see a row with:

Secret

7️⃣ Click Show
 8️⃣ Copy the long string (example):

AIzaSyBxxxxxxxxxxxxxxxxxxxxxxxx

THIS IS YOUR DATABASE SECRET

Keep it private (do not share publicly)

## WHAT YOU HAVE NOW

You should have two things saved:

### 1️⃣ Realtime Database URL

Example:

https://edge-data-filtering-default-rtdb.firebaseio.com/

### 2️⃣ Database Secret

Example:

AIzaSyBxxxxxxxxxxxxxxxxxxxx

## HOW THESE ARE USED TOGETHER

Firebase HTTP format:

https://DATABASE_URL/path.json?auth=DATABASE_SECRET

### Example:

https://edge-data-filtering-default-rtdb.firebaseio.com/test.json?auth=AIzaSyBxxxx

# STEP 10: TEST FIREBASE CONNECTION FROM PC (VERY IMPORTANT)

Before touching OKT507-C, test from PC.

### Open Terminal and run:

curl -X POST \

https://edge-data-filtering-default-rtdb.firebaseio.com/test.json \

-d '{"status":"firebase connected"}'

Now check Realtime Database → Data
 You should see:

{

"test": {

"status": "firebase connected"

}

}

If you see this → Firebase is working perfectly

Ubuntu Terminal Command: curl -X POST \

https://edge-data-filtering-default-rtdb.asia-southeast1.firebasedatabase.app/test.json \

-d '{"status":"firebase connected"}'

Secret code: Ngz9KSI3boRFGeRMU4mb5VijUETYNLOnbA5dVqis

To send Temperature data from arduino to firebase 
we need
1.APIKEY: apiKey: "AIzaSyCR_FwvqqMGctW9i6MNn4ZUAGcuIjxPAqQ"
2.Database Url :https://edge-data-filtering-default-rtdb.asia-southeast1.firebasedatabase.app/test/-OhEqfdWzZsTghqT2xMh

3.Email:teja123@gmail.com
4,Password:Votary@123

PART A: How to Get Firebase API Key

### Step A1: Open Firebase Console

- Go to  https://console.firebase.google.com

- Open your project
 edge-data-filtering

### Step A2: Go to Project Settings

- Click ⚙️ Settings (top-left)

- Click Project settings

### Step A3: Copy API Key

- You will land on General tab

- Scroll to Your apps

- You will see:

Web API Key

AIzaSyXXXXXXXXXXXX

- Copy this key

This is your API_KEY

Example:

#define API_KEY "AIzaSyXXXXXXXXXXXX"

That message is normal 
It simply means you have not added any app to your Firebase project yet.
 API Key is generated only after you add an app.

Below is the exact step-by-step fix.

# WHY THIS IS HAPPENING

Firebase shows “There are no apps in your project” because:

- Firebase project exists

- Realtime Database exists

- Authentication exists

- No app (Web / Android / iOS) is registered

👉 ESP32 uses the Web API key, so we must add a Web App.

# SOLUTION: ADD A WEB APP (REQUIRED FOR ESP32)

## STEP 1: Open Your Firebase Project

- Go to  https://console.firebase.google.com

- Select edge-data-filtering

## STEP 2: Go to Project Settings

- Click ⚙️ Settings (top-left)

- Click Project settings

## STEP 3: Add a Web App

- Scroll to Your apps

- Click the </> Web icon

## STEP 4: Register the App

- App nickname
 Example:

ESP32_Web_App

- Do NOT check “Firebase Hosting”

- Click Register app

## STEP 5: Copy the API Key

After registering, Firebase will show:

const firebaseConfig = {

apiKey: "AIzaSyXXXXXXXXXXXX",

authDomain: "...",

databaseURL: "...",

}

### COPY ONLY THIS:

apiKey: "AIzaSyXXXXXXXXXXXX"

This is your Firebase API Key

## STEP 6: Finish Setup

- Click Continue to console

Now go back to:

⚙️ Settings → Project settings → General

You will now see:

Web API Key AIzaSyXXXXXXXXXXXX

IN APP you find this code

<script type="module">

// Import the functions you need from the SDKs you need

import { initializeApp } from "https://www.gstatic.com/firebasejs/12.7.0/firebase-app.js";

// TODO: Add SDKs for Firebase products that you want to use

// https://firebase.google.com/docs/web/setup#available-libraries

// Your web app's Firebase configuration

const firebaseConfig = {

apiKey: "AIzaSyCR_FwvqqMGctW9i6MNn4ZUAGcuIjxPAqQ",

authDomain: "edge-data-filtering.firebaseapp.com",

databaseURL: "https://edge-data-filtering-default-rtdb.asia-southeast1.firebasedatabase.app",

projectId: "edge-data-filtering",

storageBucket: "edge-data-filtering.firebasestorage.app",

messagingSenderId: "799430050434",

appId: "1:799430050434:web:41e7c0cf3e23921f15ea9b"

};

// Initialize Firebase

const app = initializeApp(firebaseConfig);

</script>

3.Email
4,Password

To get this email and password follow these steps

# How to Get Firebase Email & Password (Step-by-Step)

Firebase ESP32 library uses Email/Password authentication, so we must create a user in Firebase.

## ✅ STEP 1: Open Firebase Console

- Go to https://console.firebase.google.com

- Open your project
  edge-data-filtering

## ✅ STEP 2: Enable Email/Password Sign-In

- In left menu → click Authentication

- Click Get started (if not enabled already)

- Go to Sign-in method tab

- Click Email/Password

- Enable the toggle

- Click Save

✅ Email/Password authentication is now enabled

## ✅ STEP 3: Create a Firebase User (This gives Email & Password)

- Still inside Authentication

- Click Users tab

- Click Add user

Fill details:

| Field | Example |
| --- | --- |
| Email | esp32device@gmail.com |
| Password | esp32@123 |

Use simple credentials (this is for device auth)

- Click Add user

✅ User created successfully

## ✅ STEP 4: Use These in ESP32 Code

Now you have:

### API Key

AIzaSyCR_FwvqqMGctW9i6MNn4ZUAGcuIjxPAqQ

### 🌐 Database URL (IMPORTANT: base URL only)

DO NOT include /test/-OhEqfdWzZsTghqT2xMh

Correct format:

https://edge-data-filtering-default-rtdb.asia-southeast1.firebasedatabase.app/

### Email

esp32device@gmail.com

### Password

esp32@123

Final arduino code
#include <Arduino.h>

#include <WiFi.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#include <Firebase_ESP_Client.h>

#include "addons/TokenHelper.h"

#include "addons/RTDBHelper.h"

/* ================= WiFi Credentials ================= */

#define WIFI_SSID        "iSprout-NRE"

#define WIFI_PASSWORD    "Isprout@n-202$"

/* ================= Firebase Credentials ================= */

#define API_KEY          "AIzaSyCR_FwvqqMGctW9i6MNn4ZUAGcuIjxPAqQ"

#define DATABASE_URL     "https://edge-data-filtering-default-rtdb.asia-southeast1.firebasedatabase.app/"

#define USER_EMAIL       "tejaswini.kopperla@votarytech.com"

#define USER_PASSWORD    "Votarytech@2025"

/* ================= Temperature Sensor ================= */

#define ONE_WIRE_BUS     15

#define TEMP_THRESHOLD   28.0

/* ================= Indicator Switch Pins ================= */

#define LEFT_INDICATOR_PIN   4

#define RIGHT_INDICATOR_PIN  5

/* ================= Objects ================= */

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

FirebaseData fbdo;

FirebaseAuth auth;

FirebaseConfig config;

/* ================= Timing ================= */

unsigned long lastSend = 0;

const unsigned long interval = 5000;

/* ================= Alert Tracking ================= */

int highTempCount = 0;

bool wasAboveThreshold = false;

/* ================= Diagnostics ================= */

#define DTC_BATTERY_TEMP_SENSOR_FAULT "P0A1A"

bool dtc_active = false;

unsigned long fault_start_time = 0;

const unsigned long FAULT_CONFIRM_TIME = 3000; // ms

void diagnostics_check_temperature(float temperatureC)

{

bool fault_condition =

(temperatureC == DEVICE_DISCONNECTED_C ||

temperatureC < -40.0 ||

temperatureC > 125.0);

if (fault_condition)

{

if (!dtc_active)

{

if (fault_start_time == 0)

fault_start_time = millis();

if (millis() - fault_start_time >= FAULT_CONFIRM_TIME)

{

dtc_active = true;

Serial.println("DTC SET: P0A1A - Battery Temp Sensor Fault");

Firebase.RTDB.setBool(&fbdo, "/dtc/P0A1A/status", true);

Firebase.RTDB.setString(

&fbdo,

"/dtc/P0A1A/description",

"Battery Temperature Sensor Fault");

Firebase.RTDB.setInt(

&fbdo,

"/dtc/P0A1A/timestamp",

millis());

}

}

}

else

{

fault_start_time = 0;

if (dtc_active)

{

Serial.println("DTC CLEARED: P0A1A");

Firebase.RTDB.setBool(&fbdo, "/dtc/P0A1A/status", false);

Firebase.RTDB.setString(

&fbdo,

"/dtc/P0A1A/description",

"Battery Temperature Sensor OK");

}

dtc_active = false;

}

}

void setup()

{

Serial.begin(115200);

pinMode(LEFT_INDICATOR_PIN, INPUT_PULLUP);

pinMode(RIGHT_INDICATOR_PIN, INPUT_PULLUP);

/* ---------- WiFi ---------- */

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED)

delay(500);

/* ---------- Temperature Sensor ---------- */

sensors.begin();

/* ---------- Firebase ---------- */

config.api_key = API_KEY;

config.database_url = DATABASE_URL;

auth.user.email = USER_EMAIL;

auth.user.password = USER_PASSWORD;

Firebase.begin(&config, &auth);

Firebase.reconnectWiFi(true);

}

void loop()

{

if (millis() - lastSend < interval)

return;

lastSend = millis();

/* ================= TEMPERATURE ================= */

sensors.requestTemperatures();

float temperatureC = sensors.getTempCByIndex(0);

/* Always send the temperature to Firebase, even if disconnected */

Firebase.RTDB.setFloat(&fbdo, "/sensor/temperature", temperatureC);

if (temperatureC == DEVICE_DISCONNECTED_C)

{

Serial.println("DS18B20 not detected");

Firebase.RTDB.setBool(

&fbdo,

"/diagnostics/battery_temp_sensor/status",

true);

Firebase.RTDB.setString(

&fbdo,

"/diagnostics/battery_temp_sensor/msg",

"DS18B20 not detected");

diagnostics_check_temperature(DEVICE_DISCONNECTED_C);

}

else

{

Serial.print("Temperature: ");

Serial.print(temperatureC);

Serial.println(" °C");

diagnostics_check_temperature(temperatureC);

/* Clear sensor diagnostic when OK */

Firebase.RTDB.setBool(

&fbdo,

"/diagnostics/battery_temp_sensor/status",

false);

Firebase.RTDB.setString(

&fbdo,

"/diagnostics/battery_temp_sensor/msg",

"Sensor OK");

}

/* ================= THRESHOLD ================= */

bool isAboveThreshold = (temperatureC > TEMP_THRESHOLD);

if (isAboveThreshold && !wasAboveThreshold)

highTempCount++;

wasAboveThreshold = isAboveThreshold;

if (isAboveThreshold)

{

Firebase.RTDB.setBool(

&fbdo,

"/alerts/high_temp/status",

true);

Firebase.RTDB.setFloat(

&fbdo,

"/alerts/high_temp/value",

temperatureC);

Firebase.RTDB.setString(

&fbdo,

"/alerts/high_temp/msg",

(highTempCount >= 3)

? "Service Required: Battery Temperature High"

: "Temperature crossed 28C");

}

else

{

Firebase.RTDB.setBool(

&fbdo,

"/alerts/high_temp/status",

false);

}

/* ================= INDICATORS ================= */

bool leftOn   = (digitalRead(LEFT_INDICATOR_PIN) == LOW);

bool rightOn  = (digitalRead(RIGHT_INDICATOR_PIN) == LOW);

bool hazardOn = (leftOn && rightOn);

int indicatorStatus = hazardOn ? 3 : (leftOn ? 1 : (rightOn ? 2 : 0));

Firebase.RTDB.setInt(

&fbdo,

"/vehicle/indicator/status",

indicatorStatus);

Firebase.RTDB.setBool(

&fbdo,

"/vehicle/indicator/left_indicator",

leftOn);

Firebase.RTDB.setBool(

&fbdo,

"/vehicle/indicator/right_indicator",

rightOn);

Firebase.RTDB.setBool(

&fbdo,

"/vehicle/indicator/hazard",

hazardOn);

}

}

Overall Architecture (Simple View)

DS18B20

↓

ESP32  ──(Wi-Fi)──► Firebase Realtime Database

↓

OKT507-C (Linux)

(reads alert)

↓

Display Alert

OKT507-C (wget http)

↓

PC / Laptop (curl https)

↓

Firebase (https)

OKT507-C  ──wget http──►  PC / Laptop  ──curl https──► Firebase

Bridge MUST run on your Ubuntu PC

NOT inside adb shell

NOT on OKT507-C

We can do this steps on PC terminal,finally firebase url connect and run on 
Firebase HTTP bridge running on port 8080.

Follows steps on OKT507-c board side

In firebase.sh file we can write the code

#!/bin/sh

PC_IP=192.168.140.233

URL="http://$PC_IP:8080"

echo "OKT507-C Alert Listener Started"

while true

do

DATA=$(wget -qO- "$URL")

VALUE=$(echo "$DATA" | sed -n 's/.*"value":\([0-9.]*\).*/\1/p')

if [ -z "$VALUE" ]; then

echo "No data received"

sleep 2

continue

fi

VALUE_INT=${VALUE%.*}

if [ "$VALUE_INT" -gt 28 ]; then

echo " HIGH TEMPERATURE ALERT "

echo "Temperature: $VALUE °C"

else

echo "Temperature Normal: $VALUE °C"

fi

sleep 2

done

Then follow below codes for giving permissions and run the file

After running firebase HTTP port on PC then go to OKTboard terminal