Navigation 4-wheeler:

## 1. Objective

- Evaluate available navigation/map platforms suitable for automotive use

- Check feasibility on OKT507‑C hardware and Linux environment

- Validate navigation rendering and interaction on the cluster display

- Finalize a recommended solution with clear installation and usage steps

## 2.Problem Statement – Navigation System for 4-Wheeler

Most existing navigation integrations in embedded automotive clusters are limited to basic functionality such as:

- Rendering the map on the display

- Showing current vehicle position

- Displaying turn-by-turn instructions

- Fetching routes from an online service

## 3.Initial Approaches Attempted

## Platforms Explored

### 3.1 Mapbox

Overview
Mapbox is a widely used navigation and mapping platform providing SDKs for Android, Linux, and embedded systems.

Key Features

- Automotive‑grade navigation SDK

- Online and offline map support

- Customizable UI and map styles

- Turn‑by‑turn navigation

- Supports GPU‑accelerated rendering

Pros

- Good documentation and community support

- Flexible APIs for customization

- Suitable for embedded and automotive use

Cons

Requires account creation and access token Internet required for initial setup and map downloads

### 3.2 Mappls (MapmyIndia)

Overview
Mappls is an India‑focused mapping and navigation platform with strong local data coverage.

Key Features

- Accurate Indian map data

- Navigation and routing APIs

- Android SDK support

Pros

- Excellent India‑specific map accuracy

- Automotive partnerships

Cons

- Limited support/documentation for Linux and embedded boards

- SDK access requires approvals and licensing

- Integration on OKT507‑C is not straightforward

3. 3 Platform Comparison Summary

## 4.Finalized Approach:

After evaluating both platforms, Mapbox was selected as the preferred navigation solution for the OKT507‑C based 4‑wheeler cluster.

Reasons for Selection:

- Better compatibility with Linux and embedded systems

- Strong SDK and API documentation

- High flexibility for custom cluster UI integration

- Proven use in automotive and embedded products

5. Mapbox Setup – Installation & Login Steps

### 5.1 Account Creation

- Visit the Mapbox official website

- Create a Mapbox account

- Generate an Access Token from the dashboard

### 5.2 SDK Installation (Android / Emulator)

- Create an Android project in Android Studio

- Add Mapbox dependency to build.gradle

- Add the Mapbox access token to AndroidManifest.xml

- Sync the project

### 5.3 Login & Token Configuration

- Use the generated Mapbox access token

- Configure the token in the application

- Launch the application

- Verify that maps load correctly

## 6. Android Auto Evaluation

### 6.1 Desktop Head Unit (DHU) Setup in Android Studio

The Desktop Head Unit (DHU) is used to emulate an in-vehicle infotainment system for testing Android Auto applications.

#### Prerequisites

- Android Studio installed

- Android SDK installed

- USB debugging enabled on the test device or emulator

- Android Auto compatible app

#### Steps to Install and Set Up DHU

- Open Android Studio

- Go to SDK Manager → SDK Tools tab

- Enable the following tools:

  - Android Auto Desktop Head Unit

  - Android SDK Platform-Tools

- Click Apply and complete the installation

#### Locating the DHU Binary

After installation, DHU will be available inside the SDK folder:

<Android-SDK>/extras/google/auto/desktop-head-unit

#### Running the Desktop Head Unit

- Open a terminal

- Navigate to the DHU directory

- Run the following command:

./desktop-head-unit

#### Connecting Device to DHU

- Connect an Android phone to the system via USB (USB debugging enabled)

- Run the following command to forward ports:

adb forward tcp:5277 tcp:5277

- Launch Android Auto on the phone

- The Android Auto UI will be displayed on the Desktop Head Unit window

#### Validation

- Navigation UI loads on DHU screen

- Map rendering and routing function correctly

- Touch and interaction events are reflected

### 6.2Emulator Setup

- Android Auto was configured and tested using the Android Auto Emulator

- Navigation functionality was verified with Mapbox

### 6.3 Results

- Android Auto navigation worked correctly on the emulator

- Maps, routing, and UI behaved as expected

7.Challenges Faced

During the integration of Mapbox navigation on the OKT507-C Linux board and while maintaining the Ubuntu-based system, several technical challenges were encountered. These challenges affected stability, performance, and development continuity.

#### 7. 1 Network Dependency and Token Issues

- Mapbox requires:

  - Stable internet for initial setup

  - Valid access token for map loading

#### 7.2 System Crash During Update Process

- The system crashed while performing:

  - apt upgrade

  - Kernel or graphics driver updates

Possible reasons:

- Power interruption during update

- Incompatible package versions

- Insufficient disk space during upgrade

This resulted in:

- Incomplete package installation

- Broken dependencies

- System failing to boot

8.Key Learnings:

8.1 Learning Android Studio and Android Application Development

Through this project, strong hands-on experience was gained in:

- Installing and configuring Android Studio

- Creating and managing Android projects

- Understanding the Android project structure

- Working with:

  - Activities and layouts

  - Gradle build system

  - AndroidManifest configuration

### 8.2 Understanding Mapbox Navigation SDK

The project provided practical exposure to the Mapbox platform, including:

- Creating a Mapbox account and generating access tokens

- Integrating Mapbox SDK into an Android project

- Displaying maps and handling map styles

- Implementing:

  - Current location tracking

  - Route drawing

  - Turn-by-turn navigation

9. Current Status of the POC

- Android Auto application launched correctly on the emulator

- Mapbox maps were rendered properly

- Route calculation and turn-by-turn navigation were functional

- User interaction through the Android Auto interface was stable

- When attempting to deploy the same POC on the OKT507-C Linux board, critical compatibility issues were observed.

## 10.Conclusion

## Despite successful testing and validation of the navigation Proof of Concept on the Android Auto Emulator, the solution could not be deployed on the target hardware platform.

## The primary reason for this limitation is the incompatibility between Android Auto and the OKT507-C board, which arises due to the following factors:

## Android Auto dependency on Android OS / Android Automotive OS
 Android Auto applications require a certified Android framework environment, which is not available on the OKT507-C platform.

## OKT507-C running a Linux-based operating system
 The target board operates on a standard Linux OS, and does not support the Android framework required by Android Auto.

## Lack of Android Auto certification for the hardware
 The OKT507-C board is not a Google-certified platform for Android Auto, which prevents official support and deployment.

## As a result, although the navigation application worked correctly in the emulator environment, it is not deployable on the actual target hardware.