Analytics & Alert Logic

## 1. Main Objective of Analytics & Alert Logi c

The main objective of Analytics & Alert Logic is to:

Convert raw data into meaningful insights and timely alerts to enable fast, accurate decisions.

In simple words:

- Analytics → Understand what is happening and why

- Alert Logic → Notify when immediate action is required

### Key objectives:

- Detect abnormal conditions early (faults, overheating, failures)

- Reduce manual monitoring

- Enable proactive and predictive actions

- Improve system safety, reliability, and performance

- Support real-time decision making (Edge or Cloud)

## 2. Advantages of Analytics & Alert Logic

### 1. Early Fault Detection

- Identifies issues before failure happens

- Example: Battery temperature rising beyond safe limit

### 2. Real-Time Response

- Alerts trigger immediate action

- Useful in safety-critical systems (vehicles, IoT, industrial systems)

### 3. Reduced Downtime

- Prevents sudden breakdowns

- Enables planned maintenance instead of emergency repair

### 4. Improved Safety

- Alerts protect users, equipment, and environment

- Example: Over-voltage, over-temperature alerts

### 5. Better Decision Making

- Data-driven insights instead of assumptions

- Helps engineers, operators, and management

This is work flow

Step 1: Sensor Data Collection
 First, a temperature sensor connected to the ESP32 continuously measures the battery or ambient temperature.
 These readings are generated in real time as raw sensor data.

Step 2: Threshold Defined at the Edge
 Inside the ESP32, a temperature threshold of 28 °C is already defined.
 This is important because it means the decision logic runs locally on the device, instead of sending all data to the cloud for processing.

Step 3: Data Filtering
 Now, the ESP32 performs analytics by filtering the data:

- If the temperature is below 28 °C, the data is simply ignored.
 No alert is generated and no data is sent to the cloud.
 This helps reduce network usage and cloud cost.

- If the temperature exceeds 28 °C, the edge device detects a threshold violation.
 It generates an alert flag, increments a violation counter, and prepares only this important information for transmission.

So instead of sending continuous raw data, only meaningful events are selected.

Step 4: Data Sent to Cloud (Firebase)
 Using Wi-Fi, the ESP32 sends only the alert-related data to the Firebase Realtime Database.
 The cloud stores:

- Alert flag

- Current temperature

- Threshold exceed count

This ensures the cloud receives filtered and relevant data only.

Step 5: Continuous Monitoring by Cluster
 The instrument cluster (OKT507-C) continuously monitors the alert flag and exceeds count from Firebase in real time.

Step 6: Decision Logic at Cluster Side
 To avoid false alarms, the cluster applies its own logic:
 If the temperature exceeds 28 °C more than three times, only then a service warning is triggered.
 This prevents alerts caused by short or temporary temperature spikes.

Step 7: Driver Alert Display
 Once the condition is met, the cluster displays the message:
 “Service Required: Battery Temperature High.”
 This informs the driver at the right time, helping prevent battery damage and improving vehicle safety.

Arduino Code

#include <Arduino.h>

#include <WiFi.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#include <Firebase_ESP_Client.h>

#include "addons/TokenHelper.h"

#include "addons/RTDBHelper.h"

/* ================= WiFi Credentials ================= */

#define WIFI_SSID        "iSprout-NRE"

#define WIFI_PASSWORD    "Isprout@n-202$"

/* ================= Firebase Credentials ================= */

#define API_KEY          "AIzaSyCR_FwvqqMGctW9i6MNn4ZUAGcuIjxPAqQ"

#define DATABASE_URL     "https://edge-data-filtering-default-rtdb.asia-southeast1.firebasedatabase.app/"

#define USER_EMAIL       "tejaswini.kopperla@votarytech.com"

#define USER_PASSWORD    "Votarytech@2025"

/* ================= Temperature Sensor ================= */

#define ONE_WIRE_BUS     15

#define TEMP_THRESHOLD  28.0

/* ================= Objects ================= */

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

FirebaseData fbdo;

FirebaseAuth auth;

FirebaseConfig config;

/* ================= Timing ================= */

unsigned long lastSend = 0;

const unsigned long interval = 5000;

/* ================= Analytics Tracking ================= */

int highTempCount = 0;

bool wasAboveThreshold = false;

/* ================= SETUP ================= */

void setup() {

Serial.begin(115200);

delay(1000);

/* Temperature Sensor */

sensors.begin();

Serial.println("DS18B20 Initialized");

/* WiFi */

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi Connected");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

/* Firebase */

config.api_key = API_KEY;

config.database_url = DATABASE_URL;

auth.user.email = USER_EMAIL;

auth.user.password = USER_PASSWORD;

Firebase.begin(&config, &auth);

Firebase.reconnectWiFi(true);

Serial.println("Firebase Connected");

}

/* ================= LOOP ================= */

void loop() {

if (millis() - lastSend >= interval) {

lastSend = millis();

/* ===== Read Temperature ===== */

sensors.requestTemperatures();

float temperatureC = sensors.getTempCByIndex(0);

if (temperatureC == DEVICE_DISCONNECTED_C) {

Serial.println(" DS18B20 not detected");

return;

}

Serial.print("Temperature: ");

Serial.print(temperatureC);

Serial.println(" °C");

/* ===== EDGE ANALYTICS ===== */

bool isAboveThreshold = (temperatureC > TEMP_THRESHOLD);

// Rising-edge detection (Normal → High)

if (isAboveThreshold && !wasAboveThreshold) {

highTempCount++;

Serial.print("⚠ High Temp Event Count: ");

Serial.println(highTempCount);

}

wasAboveThreshold = isAboveThreshold;

/* ===== Firebase Analytics ===== */

Firebase.RTDB.setFloat(&fbdo, "/analytics/temperature/value", temperatureC);

Firebase.RTDB.setInt(&fbdo, "/analytics/temperature/event_count", highTempCount);

Firebase.RTDB.setBool(&fbdo, "/alerts/high_temp/status", isAboveThreshold);

/* ===== Alert Messages ===== */

if (isAboveThreshold) {

Firebase.RTDB.setFloat(&fbdo, "/alerts/high_temp/value", temperatureC);

if (highTempCount >= 3) {

Firebase.RTDB.setString(

&fbdo,

"/alerts/high_temp/msg",

"Service Required: Battery Temperature High"

);

} else {

Firebase.RTDB.setString(

&fbdo,

"/alerts/high_temp/msg",

"Temperature crossed 28C"

);

}

} else {

Firebase.RTDB.setString(

&fbdo,

"/alerts/high_temp/msg",

"Temperature Normal"

);

}

}

}

## Conclusion: Analytics & Alert Logic

Analytics & Alert Logic plays a critical role in turning raw sensor data into actionable insights and timely alerts, enabling faster and smarter decision-making. By processing data at the edge and applying intelligent thresholds, it reduces unnecessary data transfer, avoids false alarms, and ensures only meaningful events are communicated.

The workflow—from sensor data collection → edge threshold checking → filtered cloud transmission → cluster-side decision → driver alert—demonstrates a real-time, efficient, and scalable system that improves safety, reliability, and performance.

Implementing this logic using platforms like ESP32 with Firebase shows how IoT edge devices can handle analytics locally, while the cloud supports monitoring and alert management. Overall, Analytics & Alert Logic enables proactive maintenance, prevents failures, and enhances user safety, making it an indispensable component in modern connected systems such as vehicles and industrial IoT.